# Private Network Service Platform - Architecture Document

Computer Networks project, Phase 1  |  Team: team07  |  Kevish, Bhawana, Abhimaan  |  4 October 2026

Domain: app.team07.test / api.team07.test. Network: Rishihood_Learner, 10.7.0.0/19, gateway 10.7.0.1. Three Macs: the DNS and edge roles are on separate Macs and both backends run on one Mac on ports 3001 and 3002.

# 1. Network topology

# 2. Machine roles and IP / service table

| Owner | Role | IP | Services / ports |
| --- | --- | --- | --- |
| Bhawana | Private DNS | 10.7.19.109 | dnsmasq, UDP 53 |
| Abhimaan | Edge: TLS terminator, reverse proxy, load balancer | 10.7.7.5 | nginx TCP 80 (redirect to HTTPS), TCP 443 (TLS, HTTP/2) |
| Kevish | Backends, test client, Wireshark | 10.7.21.121 | Backend A TCP 3001, Backend B TCP 3002 |

DNS records: app.team07.test and api.team07.test point to 10.7.7.5 (TTL 60 s). Other names are forwarded to 8.8.8.8 and 1.1.1.1. Clients on the private DNS: Kevish and Abhimaan (Wi-Fi DNS = 10.7.19.109).

# 3. Request flow, layer by layer

| Step | Layer / protocol | Evidence (CN-phase-1.pcapng) |
| --- | --- | --- |
| 1-2 | DNS over UDP 53 | Frames 3-4: 10.7.21.121:51288 to 10.7.19.109:53; answer A 10.7.7.5 |
| - | ARP / Ethernet | Frames 5, 7: who-has 10.7.7.5, reply 72:94:6f:7b:aa:c9; frames go directly to the host, not the gateway |
| 3 | TCP three-way handshake | Frames 6, 8, 9: 59877 to 443, SYN, SYN-ACK, ACK |
| 4 | TLS 1.2 handshake | Frames 10-17: Client Hello (SNI, ALPN), Server Hello and Certificate (issuer team07 Local Root CA), key exchange, Finished |
| 5, 8 | HTTP/2 inside TLS | Frames 18-34: Application Data (encrypted) |
| 6-7 | Plain HTTP/1.1, edge to backend | Frames 23-31: 10.7.7.5:52981 to 10.7.21.121:3001, GET /api/status with X-Forwarded-* headers |

TLS terminates at nginx. The edge opens a second TCP connection to the chosen backend. Backends see the edge IP (from=10.7.7.5); the real client IP is only in X-Forwarded-For.

# 4. OSI / TCP-IP mapping

| OSI | TCP/IP | In this project |
| --- | --- | --- |
| 7 Application | Application | HTTP/1.1, HTTP/2, DNS, Cache-Control, ETag, X-Backend |
| 6 Presentation | Application | TLS encryption and certificates (OpenSSL CA, nginx ssl) |
| 5 Session | Application | TLS session, ALPN negotiation |
| 4 Transport | Transport | TCP 443, 3001, 3002 (handshake, Seq/Ack, Win, SACK); UDP 53 |
| 3 Network | Internet | IPv4 10.7.0.0/19, ICMP ping (ttl=64) |
| 2 Data link | Link | Wi-Fi frames, MAC addresses, ARP |
| 1 Physical | Link | Wi-Fi radio |

# 5. Cloud equivalents

| Our component | Cloud equivalent |
| --- | --- |
| dnsmasq host-records | Amazon Route 53 private hosted zone |
| nginx TLS termination, round-robin, retry | Application Load Balancer with ACM certificate and target-group health checks |
| Backend A / B | EC2 or ECS targets in a target group |
| Own root CA and server certificate | AWS Certificate Manager or a public CA |
| max_fails, fail_timeout, proxy_next_upstream | ALB health checks and automatic deregistration |
| nginx 502 when both backends are down | ALB 502/503 with no healthy target |
| Wireshark on a client | VPC Flow Logs, traffic mirroring |

# 6. Design decisions

.test is reserved for private use; public DNS returns NXDOMAIN from the root zone (.local belongs to mDNS).

Caching uses backend Cache-Control and ETag, not nginx proxy_cache, so load-balancer alternation stays visible. Identical content gives identical ETags, so 304 works from either backend.

Both names point at the edge; clients never learn backend IPs.

Single points of failure: the DNS Mac and the edge Mac (addressed in Phase 2).

# 7. Failure demonstrations (summary)

| # | Fault | Layer | Result | curl exit |
| --- | --- | --- | --- | --- |
| H1 | Wrong resolver 8.8.8.8 | DNS | NXDOMAIN; ping by IP works | 6 |
| H2 | DNS record points to wrong IP | TLS | TCP connects, certificate not trusted | 60 |
| H3 | Backend A stopped | App tier | Answers only from B, error.log shows refused :3001 | 0 |
| H4 | Both backends stopped | App tier | HTTP/2 502 from nginx | 0 (502) |
| H5 | Wrong port 8443 | TCP | Connection refused, host reachable | 7 |

# 8. Incidents and fixes

No route to host toward Kevish (first ping round): an ARP failure, cleared by reconnecting Wi-Fi; all six ping directions then passed with ttl=64.

nginx refused on port 80: a leftover nginx from an earlier project held ports 8080/443 and its configuration did not include servers/*. Fixed with a clean nginx.conf and a root-owned brew service.

# 9. Deliverables map

Configuration bundle: dns/, nginx/, certs/. Backend source: backend/. Evidence: evidence/A-lan to H-failures (see evidence/INDEX.md). Operations: docs/SETUP.md, docs/RUNBOOK.md, docs/VIVA.md.